#include <IRremote.h>

#include <Stepper.h>

// constants that won't change:

const int buttonPin = 13;     // the number of the pushbutton pin

const int RECV_PIN = 11;      // the IR receive pin

const int stepsPerRevolution = 2048;  //number of steps per revolution of specific stepper motor

Stepper myStepper(stepsPerRevolution, A0, A1, A2, A3);  // setup pins 8-11 as outputs to control stepper driver

int num_array[10][7] = {  { 0,0,0,0,0,0,1 },    // 0

{ 1,0,0,1,1,1,1 },    // 1

{ 0,0,1,0,0,1,0 },    // 2

{ 0,0,0,0,1,1,0 },    // 3

{ 1,0,0,1,1,0,0 },    // 4

{ 0,1,0,0,1,0,0 },    // 5

{ 0,1,0,0,0,0,0 },    // 6

{ 0,0,0,1,1,1,1 },    // 7

{ 0,0,0,0,0,0,0 },    // 8

{ 0,0,0,1,1,0,0 }};   // 9

long one = 0xFF30CF;   // hexadecimal codes receives from IR remote

long two = 0xFF18E7;

long three = 0xFF7A85;

long play = 0xFFC23D;

long pause = 0xFF6897;

long stopAndCal = 0xFF629D;

// variables will change:

int buttonState = 0;         // variable for reading the pushbutton status

int count = 1;               // Count the button presses, starting at 1 to default to mode 1

int stepCount = 0;  //initial step count of motor at 0

int start = 0;  // initalize mode starter and defaults it to not run any modes

//function header

void Num_Write(int);

// initialize functions

IRrecv irrecv(RECV_PIN);  // receive IR hex text from the RECV_PIN

decode_results results;   // write the IR received results

void setup() {

pinMode(buttonPin, INPUT); // initialize the push button pin read as an input

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

pinMode(8, OUTPUT);

Serial.begin(9600); //

irrecv.enableIRIn(); //

//Timer1 Interrupt Setup for 10 Hz

cli();  //stop interrupts

TCCR1A = 0; //set entire TCCR1A register to 0

TCCR1B = 0; //set entire TCCR1B register to 0

TCNT1 = 0;  //initialize counter value at 0

OCR1A = 6249; // = (16,000,000/(256x10))-1, <65536 (16 bit)

TCCR1B |= (1 << WGM12); //turn on clear timer on compare match

TCCR1B |= (1 << CS12) | (0 << CS10);  //set CS10 and CS12 for 256 prescaler

TIMSK1 |= (1 << OCIE1A);  //enable timer compare interrupt

sei();  //allow interrupts

}

// Timer1 interrupt sequence to look for pause and stop pushes while in mode routine

ISR(TIMER1_COMPA_vect){ //interrupt at 10 Hz frequency to look for pause or stop button pushes

if (irrecv.decode(&results)) {      // look for IR signal, if received, write to results

Serial.println(results.value, HEX);

irrecv.resume(); // Receive the next value

}

if (results.value == pause && start ==1){ //look to see if pause was pushed and was already in mode routine

while (results.value == pause){  //look for IR codes until play is pushed again, resume routine where left off

Serial.println("Reached While Loop");

Serial.println(results.value);

if(irrecv.decode(&results)){      // look for IR signal, if received, write to results

Serial.println(results.value, HEX);

irrecv.resume(); // Receive the next value

Serial.print("Results Value Variable: ");

Serial.println(results.value);

}

}

}

//INSERT STOP AND CALIBRATE CODE HERE ONCE CALIBRATION SEQUENCE CONFIRMED

/*

* if(results.value == stopAndCal && start == 1){ // look to see if stop was pushed and was already in mode routine

*    results.value = 0;  //reset IR code

*    start = 0;  //reset mode start trigger

*

*    DO CALIBRATION STEPS HERE

*  }

*/

}

void loop(){

buttonState = digitalRead(buttonPin); // read the state of the push button and set result equal to buttonState

if (buttonState == HIGH) {   //check to see if the push button is pressed

count++;    // increase the value of count by 1

delay(250); // wait .25 sec to ensure only 1 button press is registered and debounce switch

}

if (count == 4) { // check to see if the button reached the max number of modes

count = 1;  // reset the count back to mode 1

}

//Look for IR signal, write the received hex code to results.value, compare the hex code to known values, adjust count or play trigger accordingly

if (irrecv.decode(&results)) {      // look for IR signal, if received, write to results

Serial.println(results.value, HEX);

irrecv.resume(); // Receive the next value

}

if(results.value == one) {

count = 1;

}

if(results.value == two) {

count = 2;

}

if(results.value == three) {

count = 3;

}

if(results.value == play) {

start = 1;

}

//Set the seven segment display value to the count from either the push button or the IR

if(count == 1) {

int pin= 2;

for (int j=0; j < 7; j++) {

digitalWrite(pin, num_array[1][j]);  // display 1 on 7 segment display

pin++;

}

}

if(count == 2) {

int pin= 2;

for (int j=0; j < 7; j++) {

digitalWrite(pin, num_array[2][j]);    // display 2 on 7 segment display

pin++;

}

}

if(count == 3) {

int pin= 2;

for (int j=0; j < 7; j++) {

digitalWrite(pin, num_array[3][j]);  // display 3 on 7 segment display

pin++;

}

}

//Stepper motor code cases are controlled by the count, the code only runs if the start trigger value is 1, resets start trigger to 0 to only run routine once

if(start == 1){

switch(count){

case 1: {

delay(1000);  // pause 1 second from time of switching mode until routine begins

myStepper.setSpeed(16); //set the speed

myStepper.step(2048);  //index stepper motor positively

delay(1000);

myStepper.step(-2048); //index stepper motor negatively

start = 0; // reset the mode starter to 0 so the mode code only runs once unless retriggered

results.value = 0;  // reset the IR code so it doesn't begin to play again

break;

}

case 2: {

delay(1000);  // pause 1 second from time of switching mode until routine begins

myStepper.setSpeed(random(5,15)); //set the speed

delay(1000);

myStepper.step(random(50,2048));  //index stepper motor positively

myStepper.setSpeed(random(5,15)); //set the speed

delay(1000);

myStepper.step(-random(50,2048)); //index stepper motor negatively\

start = 0; // reset the mode starter to 0 so the mode code only runs once unless retriggered

results.value = 0;  // reset the IR code so it doesn't begin to play again

break;

}

case 3: {

delay(1000);  // pause 1 second from time of switching mode until routine begins

myStepper.setSpeed(16); //set the speed

myStepper.step(4096);  //index stepper motor positively

delay(1000);  //pause for 1 second

myStepper.step(-4096); //index stepper motor negatively

delay(1000);

start = 0; // reset the mode starter to 0 so the mode code only runs once unless retriggered

results.value = 0;  // reset the IR code so it doesn't begin to play again

break;

}

}

}