116.000MHz high precision quartz  TCXO specification

| parameter | character | technical parameters | unit |
| --- | --- | --- | --- |
| output | working frequency | 116.0000 | MHz |
|  | frequency accuracy | ±0.1Max，Normal temperature 25°C | ppm |
|  | Waveform (square wave/sine wave) | Sine waves |  |
|  | phase noise | -105dbc/100Hz -125dbc/1KHz -135dbc/10KHz |  |
|  | Increase/decrease time (square)） | 10 | ns |
| temperature characteristic | temperature characteristic | ±0.5/-20-70°C | ppm |
| aging rate | aging rate/year | 1 Max | ppm |
| power supply | supply voltage | 3-5v | VDC |
|  | working current | 30 Max | mA |
|  | Load regulation | ±0.2 Max | ppm |
| other specifications | operating temperature | -20~+70 | °C |
|  | storage temperature | -40~+85 | °C |
|  | Overall dimensions | 20.3*12.5*8 | mm |
|  | start time to oscillate | 5msMax |  |
|  | output level“0” |  |  |
|  | output level“1” |  |  |
|  | output symmetry | 1v Vpp |  |
|  | output load | 50 ohm |  |

size

pin functions

1: NC

7: GND

8: OUT

14:VC（3v-5v）