Setup the ESP8266 Core

- Setting up ESP8266 Core for Arduino IDE

Arduino IDE do not have a native support for NodeMCU borads but, we can integrate ESP8266 Core for Arduino IDE by following the process given here.

- Open Arduino IDE

- Go to File->Preferences

- In “Additional Boards Manager URLs:” paste this URL

- http://arduino.esp8266.com/stable/package_esp8266com_index.json

- Open Boards Manager from Tools

- Serach for esp8266 and click on install button

- After installation of Ardunio Core for ESP8266 go to Tools and select board NodeMCU 1.0 (ESP-12E Module)

- Make ssure you have correct settings for CPU frequency, Flash Size, Upload Speed and Port(the port number may vary depending on the PC you are currently working on).

Process:
Select board: Arduino Uno Wifi
Load example: Tool/ESPRecovery from: https://github.com/arduino-org/arduino-library-arduino-wifi
Upload to the board.
Unplug Arduino Uno Wifi
Plug Arduino Uno Wifi but pressing at same time: Bootloader button

Now you are programming directly the ESP8266 because Arduino side is working as a simple FTDI
Load your sketch for the ESP8266 side in Arduino IDE. Note that you must communicate through the Serial with Arduino side. SerialEvent Example can be a good start point
Select Board: Generic ESP8266, DIO, Flash 4M, Reset ck, Speed 9600, COM (your number)
Upload the sketch, wait done.

Unplug and plug the board
Select board: Arduino Uno Wifi
Load your sketch for the Arduino side, you can communicate in both side as in Tool/ESPRecovery and must receive orders as in SerialEvent Example. You should also neglect the initial garbage and treat the possible communications of the ESP8266 side, like for example the WDT.
Upload to the board.

- Installing pub-sub library for MQTT communications

- Copy pubsubclient-master folder and paste it in libraries folder present in Arduino IDE installation directory.

- What is the code?

As NodeMCU is based on ESP8266 and ESP8266 has Arduino Core so same like Arduino, ESP8266 code consists of two primitive functions i.e, setup() and loop()

- Open Arduino IDE

- Use the following code.

#include <ESP8266WiFi.h>

#include <PubSubClient.h>

const char *ssid =	"abc";

const char *pass =	"abc123";

const char *mqtt_server = "broker.hivemq.com";

const int mqtt_port = 1883;

const char *mqtt_user = "testUser";

const char *mqtt_pass = "abc123";

const char *mqtt_client_name = "12312312332212";// any random alphanumeric stirng

#define BUFFER_SIZE 100

String incoming="";

WiFiClient wclient;

PubSubClient client(wclient, mqtt_server,mqtt_port);

void callback(const MQTT::Publish& pub) {

// handle message arrived

Serial.print(pub.topic());

Serial.print(" => ");

if (pub.has_stream()) {

uint8_t buf[BUFFER_SIZE];

int read;

while (read = pub.payload_stream()->read(buf, BUFFER_SIZE)) {

Serial.write(buf, read);

}

pub.payload_stream()->stop();

Serial.println("");

} else

{

Serial.println(pub.payload_string());

}

////////////////////////////////////////////

incoming=String(pub.payload_string());

Serial.println(pub.payload_string());

Serial.println(incoming);

pinMode(16,OUTPUT);

if(incoming.indexOf("on")!=-1){

digitalWrite(16,HIGH);

}

else if(incoming.indexOf("off")!=-1){

digitalWrite(16,LOW);

}

}

void setup() {

// Setup console

Serial.begin(115200);

delay(10);

Serial.println();

Serial.println();

}

void loop() {

if (WiFi.status() != WL_CONNECTED) {

Serial.print("Connecting to ");

Serial.print(ssid);

Serial.println("...");

WiFi.begin(ssid, pass);

if (WiFi.waitForConnectResult() != WL_CONNECTED)

return;

Serial.println("WiFi connected");

}

if (WiFi.status() == WL_CONNECTED) {

if (!client.connected()) {

Serial.println("Connecting to MQTT server");

if (client.connect(MQTT::Connect(mqtt_client_name)

.set_auth(mqtt_user, mqtt_pass))) {

Serial.println("Connected to MQTT server");

client.set_callback(callback);

client.subscribe("a/b");//topic

} else {

Serial.println("Could not connect to MQTT server");

}

}

if (client.connected())

client.loop();

}

}

- Open the MQTT Android app or MQTTLens and use broker url: Broker.hivemq.com

- Port: 1883

- In message send “on” or “off” to turn the LED on or off.

- Open serial monitor of Arduino IDE in Tools->Serial Monitor to see the incoming data.

- Open source broker HiveMQ is being used in this tutorial

- To analyze the data coming in and going out the broker, open this link:

- http://www.mqtt-dashboard.com/