Here is the relevant code:

[code]

/* This sketch controls a servo-controlled Whack a Mole game with 3 servos and 3 push buttons and a starter button with an ARDUINO UNO

*/

#include<Servo.h> // include Servo library

int heads = 0;

int inPin1 = 1; // the switch is wired to Arduino D1 pin

int inPin2 = 2; // the switch is wired to Arduino D2 pin

int inPin3 = 4; // the switch is wired to Arduino D4 pin

int inPin4 = 7; // the switch is wired to Arduino D7 pin

int reading1; // the current reading from the input pin 1 Swtich A

int reading2; // the current reading from the input pin 2 Switch B

int reading3; // the current reading from the input pin 4 Switch C

int reading4; // the current reading from the input pin 7 Switch to initiate heads moving

int servoPosition = 90;     // the current angle of the servo - starting at 90.

int servoSlowInterval = 80; // millisecs between servo moves

int servoFastInterval = 10;

int servoInterval = servoSlowInterval; // initial millisecs between servo moves

int servoDegrees = 2;       // amount servo moves at each step

//    will be changed to negative value for movement in the other direction

Servo myservoA; // create servo motor object A

Servo myservoB; // create servo motor object B

Servo myservoC; // create servo motor object C

void PushButtonFourLoop ();

void TestButtonOne();

void TestButtonTwo();

void TestButtonThree();

bool ButtonOnePressed = false;

bool ButtonTwoPressed = false;

bool ButtonThreePressed = false;

byte ButtonState1 = LOW;  // used to record whether the Button One is pushed or not, LOW is not

byte ButtonState2 = LOW;  // used to record whether the Button One is pushed or not, LOW is not

byte ButtonState3 = LOW;  // used to record whether the Button One is pushed or not, LOW is not

byte ButtonState4 = LOW;  // used to record whether the Button One is pushed or not, LOW is not

unsigned long currentMillis = 0;    // stores the value of millis() in each iteration of loop()

unsigned long previousButtonOneMillis = 0;  // will store last time Button was updated

unsigned long previousButtonTwoMillis = 0;  // will store last time Button was updated

unsigned long previousButtonThreeMillis = 0;  // will store last time Button was updated

unsigned long previousButtonFourMillis = 0;  // will store last time Button was updated

unsigned long previousServoMillis = 0; // the time when the servo was last moved

const int Button_One_Interval = 2500;  //Button A Interval led_A_Interval

const int Button_Two_Interval = 2500;  //Button B Interval led_A_Interval

const int Button_Three_Interval = 2500;  //Button C Interval led_A_Interval

const int Button_Four_Interval = 3000;  //Button C Interval led_A_Interval

const int buttonInterval = 50; // number of millisecs between button readings

const int servoMinDegrees = 55; // the limits to servo movement

const int servoMaxDegrees = 135;

//=========================================================================================================

void setup() {

Serial.begin(9600);

Serial.println("This is the main program, doing several things at once");  // so we know what sketch is running

myservoA.attach(3); // attach servo motor to pin 3 of Arduino

myservoB.attach(5); // attach servo motor to pin 5 of Arduino

myservoC.attach(6); // attach servo motor to pin 6 of Arduino

pinMode(inPin1, INPUT); // make pin 1 an input

pinMode(inPin2, INPUT); // make pin 2 an input

pinMode(inPin3, INPUT); // make pin 4 an input

pinMode(inPin4, INPUT); // make pin 7 an input

myservoA.write(55); // servo motor Lowers Head A

myservoB.write(55); // servo motor Lowers Head B

myservoC.write(55); // servo motor Lowers Head C

reading1 = digitalRead(inPin1); // store digital data in variable

reading2 = digitalRead(inPin2); // store digital data in variable

reading3 = digitalRead(inPin3); // store digital data in variable

reading4 = digitalRead(inPin4); // store digital data in variable

}

//==========

void loop() {

Serial.println("Starting main loop");

currentMillis = millis();   // capture the latest value of millis()

// this is equivalent to noting the time from a clock

// use the same time for Buttons to keep them synchronized

UpdateButtonState1 ();

UpdateButtonState2 ();

UpdateButtonState3 ();

UpdateButtonState4 ();

PushButtonFourLoop ();

TestButtonOne ();

TestButtonTwo ();

TestButtonThree ();

Serial.println("Ending main loop");

}

//========

void UpdateButtonState1 () {

if (ButtonState1 == LOW) {

// if Button ONE is off, we must wait for the interval to expire before turning it on

if (currentMillis - previousButtonOneMillis >= Button_One_Interval) {

// time is up, so change the state to HIGH

ButtonState1 = HIGH;

// and save the time when we made the change

previousButtonOneMillis += Button_One_Interval;

}

}

else {  // i.e. if Button ONE is HIGH

// if the Button ONE is on, we must wait for the duration to expire before turning it off

if (currentMillis - previousButtonOneMillis >= Button_One_Interval) {

// time is up, so change the state to LOW

ButtonState1 = LOW;

// and save the time when we made the change

previousButtonOneMillis += buttonInterval;

}

}

}

//========

void UpdateButtonState2 () {

if (ButtonState2 == LOW) {

// if Button TWO is off, we must wait for the interval to expire before turning it on

if (currentMillis - previousButtonTwoMillis >= Button_Two_Interval) {

// time is up, so change the state to HIGH

ButtonState2 = HIGH;

// and save the time when we made the change

previousButtonTwoMillis += Button_Two_Interval;

}

}

else {  // i.e. if Button TWO is HIGH

// if the Button TWO is on, we must wait for the duration to expire before turning it off

if (currentMillis - previousButtonTwoMillis >= Button_Two_Interval) {

// time is up, so change the state to LOW

ButtonState2 = LOW;

// and save the time when we made the change

previousButtonTwoMillis += buttonInterval;

}

}

}

//=========

void UpdateButtonState3 () {

if (ButtonState3 == LOW) {

// if Button THree is off, we must wait for the interval to expire before turning it on

if (currentMillis - previousButtonThreeMillis >= Button_Three_Interval) {

// time is up, so change the state to HIGH

ButtonState3 = HIGH;

// and save the time when we made the change

previousButtonThreeMillis += Button_Three_Interval;

}

}

else {  // i.e. if Button Three is HIGH

// if the Button Three is on, we must wait for the duration to expire before turning it off

if (currentMillis - previousButtonThreeMillis >= Button_Three_Interval) {

// time is up, so change the state to LOW

ButtonState3 = LOW;

// and save the time when we made the change

previousButtonThreeMillis += buttonInterval;

}

}

}

//==========

void UpdateButtonState4 () {

if (ButtonState4 == LOW) {

// if Button Four is off, we must wait for the interval to expire before turning it on

if (currentMillis - previousButtonFourMillis >= Button_Four_Interval) {

// time is up, so change the state to HIGH

ButtonState4 = HIGH;

// and save the time when we made the change

previousButtonFourMillis += Button_Four_Interval;

}

}

else {  // i.e. if Button Four is HIGH

// if the Button Three is on, we must wait for the duration to expire before turning it off

if (currentMillis - previousButtonFourMillis >= Button_Four_Interval) {

// time is up, so change the state to LOW

ButtonState4 = LOW;

// and save the time when we made the change

previousButtonFourMillis += buttonInterval;

}

}

}

//=========

void TestButtonOne () {

if (millis() - previousButtonOneMillis >= Button_One_Interval) {

if (reading1 == HIGH) { // check to see if switch ONE has been pushed

//Raise Head ONE

Serial.println("The first button has been pressed");

if (currentMillis - previousServoMillis >= servoInterval) {

// its time for another move

previousServoMillis += servoInterval;

servoPosition = servoPosition + servoDegrees; // servoDegrees might be negative

if (servoPosition <= servoMinDegrees) {

// when the servo gets to its minimum position change the interval to change the speed

if (servoInterval == servoSlowInterval) {

servoInterval = servoFastInterval;

}

else {

servoInterval = servoSlowInterval;

}

}

if ((servoPosition >= servoMaxDegrees) || (servoPosition <= servoMinDegrees))  {

// if the servo is at either extreme change the sign of the degrees to make it move the other way

servoDegrees = - servoDegrees; // reverse direction

// and update the position to ensure it is within range

servoPosition = servoPosition + servoDegrees;

}

// make the servo move to the next position

myservoA.write(servoPosition); // servo motor rotates to Raise Head One

// and record the time when the move happened

}

previousButtonOneMillis += Button_One_Interval;

ButtonState1 = ! ButtonState1;

}

}

Serial.println("This is a function testing for a button push of Switch A");

}

//========

void TestButtonTwo () {

if (millis() - previousButtonTwoMillis >= Button_Two_Interval) {

if (reading2 == HIGH) { // check to see if switch TWO has been pushed

//Raise Head TWO

Serial.println("The second button has been pressed");

if (currentMillis - previousServoMillis >= servoInterval) {

// its time for another move

previousServoMillis += servoInterval;

servoPosition = servoPosition + servoDegrees; // servoDegrees might be negative

if (servoPosition <= servoMinDegrees) {

// when the servo gets to its minimum position change the interval to change the speed

if (servoInterval == servoSlowInterval) {

servoInterval = servoFastInterval;

}

else {

servoInterval = servoSlowInterval;

}

}

if ((servoPosition >= servoMaxDegrees) || (servoPosition <= servoMinDegrees))  {

// if the servo is at either extreme change the sign of the degrees to make it move the other way

servoDegrees = - servoDegrees; // reverse direction

// and update the position to ensure it is within range

servoPosition = servoPosition + servoDegrees;

}

// make the servo move to the next position

myservoB.write(servoPosition); // servo motor rotates to Raise Head Two

// and record the time when the move happened

}

previousButtonTwoMillis += Button_Two_Interval;

ButtonState2 = ! ButtonState2;

}

}

Serial.println("This is a function testing for a button push of Switch B");

}

//========

void TestButtonThree () {

if (millis() - previousButtonThreeMillis >= Button_Three_Interval) {

if (reading2 == HIGH) { // check to see if switch Three has been pushed

//Raise Head Three

Serial.println("The third button has been pressed");

if (currentMillis - previousServoMillis >= servoInterval) {

// its time for another move

previousServoMillis += servoInterval;

servoPosition = servoPosition + servoDegrees; // servoDegrees might be negative

if (servoPosition <= servoMinDegrees) {

// when the servo gets to its minimum position change the interval to change the speed

if (servoInterval == servoSlowInterval) {

servoInterval = servoFastInterval;

}

else {

servoInterval = servoSlowInterval;

}

}

if ((servoPosition >= servoMaxDegrees) || (servoPosition <= servoMinDegrees))  {

// if the servo is at either extreme change the sign of the degrees to make it move the other way

servoDegrees = - servoDegrees; // reverse direction

// and update the position to ensure it is within range

servoPosition = servoPosition + servoDegrees;

}

// make the servo move to the next position

myservoC.write(servoPosition); // servo motor rotates to Raise Head Three

// and record the time when the move happened

}

previousButtonThreeMillis += Button_Three_Interval;

ButtonState3 = ! ButtonState3;

}

}

Serial.println("This is a function testing for a button push of Switch C");

}

//========

void PushButtonFourLoop () {

Serial.println("This is a function testing for a button push of Switch D");

if (currentMillis - previousButtonFourMillis >= Button_Four_Interval) {

if (reading4 == HIGH) {//Move Heads

Serial.println("The fourth button has been pressed");

for (int i = 1; i < 3; i++) {

//MOVES Servos

if (currentMillis - previousServoMillis >= servoInterval) {

// its time for another move

previousServoMillis += servoInterval;

servoPosition = servoPosition + servoDegrees; // servoDegrees might be negative

if (servoPosition <= servoMinDegrees) {

// when the servo gets to its minimum position change the interval to change the speed

if (servoInterval == servoSlowInterval) {

servoInterval = servoFastInterval;

}

else {

servoInterval = servoSlowInterval;

}

}

if ((servoPosition >= servoMaxDegrees) || (servoPosition <= servoMinDegrees))  {

// if the servo is at either extreme change the sign of the degrees to make it move the other way

servoDegrees = - servoDegrees; // reverse direction

// and update the position to ensure it is within range

servoPosition = servoPosition + servoDegrees;

}

// make the servo move to the next position

myservoA.write(servoPosition);

myservoB.write(servoPosition);

myservoC.write(servoPosition); // servo motor rotates to Raise Head Three

// and record the time when the move happened

}

TestButtonOne ();

TestButtonTwo ();

TestButtonThree ();

if (ButtonOnePressed == true) {

ButtonOnePressed = false;

break;

}

if (ButtonTwoPressed == true) {

ButtonOnePressed = false;

break;

}

if (ButtonThreePressed == true) {

ButtonOnePressed = false;

break;

}

}

}

previousButtonFourMillis += Button_Four_Interval;

ButtonState4 = ! ButtonState4;

}

}

//===========

[/code]