Components used

My green house is made of double wall poly carbonate sheet of GE make.  size of green house is 
8 feet  x 4feet x 4feet, due to green house effect ,inside temperature reaches upto 55-degree Celsius.

heat sink for hot and cold side size 300 mm x 90 mm x 20 mm(fins hight)

Tec module used 40 mm x 40 mm size ( 12 706 type) 3 pcs qty

Fan size (80 mm x 80 mm x 40 mm) high speed type Air delivery upto 8 feets qty 6 pcs

What I have Tried - Assembling

TEC modules   3 pcs are mounted on Flat side of heat sink  on equal distances using high quality silicon heat sink paste on both side of TEC module.

Both heat sinks are mounted back to back on Hot/Cold side of TEC module,high quality heat sink paste is applied on both side of TEC module .Fans are facing outside. both heat sinks are fixed together using 3mm screws and nuts.

To utilise the air delivery of the fans the metal enclose of 80 mm height is made of GI sheet, both 300 mm x 90 mm side are open. three fans are to be mounted on one side of encloser. Two fans are mounted to suck the cabin air and throw on cooled side of heat sink ,for third fan A small partition ,up to fins of heat sink , is created  to through the cooled air to cabin. Other side of the enclose is mounted on 300 mm x 90 mm  cool side of the heat sink.

Similar type of cabin and fans are mounted on hot side of the heat sink.

For circulating the cold air in the cabin, a small cavity is made by cutting the poly carbonate sheet ( size of window is (300 mm x 90 mm).

Cold side of heat sink with fans are kept in side the cabin for cooling the air .and hot air side of heat sink and fans are kept out side the the green house, to through the hot air out side.

Solar Energy/Battery based cooling system for my green house

Using: 12-volt panel and rechargeable car battery

Problem Statement

We are trying to maintain a temperature of up to 25-degree Celsius inside the Green House which is usually at around 55-Degree Celsius, as the project has to be solar operated / battery operated the challenge even gets tougher.

I have made a cooling system using an array of 3 TEC Modules (Thermo Electric Cooler Device),I have attached the images of prototype.

Currently the outlet air Temperature at Cool side is 15 to 18 degree Celsius and Hot Air temperature is around 32 Degree Celsius, tested in a room which is maintained at 25 Degree Celsius. I have shared the details of How it has been constructed.

Are there people who have made or heard of anything similar, My major challenge is the Cooling System and its design. Currently I am open to any input, as I have decided to make this Idea.