MS10 soil moisture & temperature sensor User’s manual

V1.0

Index

1 Features	错误！未定义书签。

2 technical parameter	错误！未定义书签。

3 Connection	错误！未定义书签。

3.Connection guide	错误！未定义书签。

4 Package size, Ordering info, and installation guide	错误！未定义书签。

4.1  Package size	错误！未定义书签。

4.2 Ordering information	错误！未定义书签。

4.3 Installation guide	错误！未定义书签。

5 The conversion for soil moisture volume percentage，temperature and output	错误！未定义书签。

6 Modbus Protocol	错误！未定义书签。

6.1 Serial comm. params	错误！未定义书签。

6.2 Modbus register	错误！未定义书签。

6.3 Detail of Modbus register	错误！未定义书签。

6.4 CRC16 validation and sample code	错误！未定义书签。

7 User software	错误！未定义书签。

7.1 software installation	错误！未定义书签。

7.2 software operates	错误！未定义书签。

1 Features

This MS10 soil moisture & temperature sensor is provided with high accurate and high sensitive. By measuring the dielectric constant of the reaction of soil, soil direct stable real moisture content.This MS10 soil moisture & temperature sensor can measure the volume of soil moisture.the soil moisture measurement method is in line with international standards at present.Apply to the soil moisture monitoring, scientific experiment, water-saving irrigation, greenhouse vegetables, flowers, grass, soil, plant cultivation, measured speed of sewage treatment, grain storage, greenhouse control, precision agriculture, at the same time in the measurement of water, weather and particulate matter content.

- High measuring accuracy, fast response, good interchangeability

- Good sealing performance, corrosion resistance, long buried in soil using

- The flame retardant epoxy resin curing, completely waterproof, can withstand the strong external shocks

- Needle using quality materials, can withstand long-term electrolysis, can withstand the corrosion of acid and alkali in the soil

- High measuring precision, reliable performance, the effects of soil salt content is small, suitable for all kinds of soil

- Having power reverse protection

2 Technical parameter

| technical parameter |  |  |  |
| --- | --- | --- | --- |
| The signal output type | Output voltage 0-2V output impedance <1Kohm | current output 4-20mA load resistance <500ohm | RS485 interface, Modbus protocol |
| Power supply voltage | 5-30V/DC | 12-30V/DC | 5-30V/DC |
| Maximum power dissipation | 40mA@24V DC | 80mA@24V DC (When the two current output channel is 20mA) | 30mA@24V DC |
| Soil moisture measurement area | The probe as the center, the diameter of 7cm, height of 7cm cylinder |  |  |
| Response time (moisture and temperature) | Less than 1 second |  |  |
| Soil moisture measurement range | Optional range: 0-30% volumetric moisture content 0-50% volumetric moisture content 0-100% volumetric moisture content |  |  |
| Soil moisture measurement accuracy | 0-53% range is ± 3%; 53-100% range is ± 5% |  |  |
| Soil temperature measuring range | -40~80℃ |  |  |
| The measurement precision of soil temperature | ±0.5℃ |  |  |
| Protection grade | IP68 |  |  |
| Operating environment | -40~85℃ |  |  |
| The storage environment | -40~85℃ |  |  |
| The material of the probe | Food grade stainless steel |  |  |
| Sealing material | The black flame retardant epoxy resin |  |  |
| Installation | All buried or probe into all of the measured medium |  |  |

3 Connection

3.1 Connection Guide

| Model | Wiring diagram |
| --- | --- |
| Voltage output type | Red (V+): Power Supply Black (G): power ground Blue (O1): the output signal (temperature) Brown (O2): the output signal (moisture) Yellow: do not connect White: do not connect Green: do not connect |
| Current output type | Red (V+): Power Supply Black (G): power ground Blue (O1): the output signal (temperature) Brown (O2): the output signal (moisture) Yellow: do not connect White: do not connect Green: do not connect |
| RS485 interface type The Modbus protocol | Red (V+): Power Supply. Black (G): power ground Blue (O1): do not connect Brown (O2): do not connect Yellow (T+): RS485+/A/T+ White (T-): RS485-/B/T- Green (SET): V+ (power) when boot module into the "setting mode". Not connected or connected with the G (power) when boot into "mode of operation". Module configuration parameters such as Modbus address, baud rate, parity, communication protocol is composed of module inside the EEPROM (power down storage device stores). The specific configuration sometimes forget these parameters that cannot communicate with the module. In order to prevent this problem, the module has a special mode called "mode". When the module is based on the "mode" electric start, the module communicates with the following parameters: Fixed Modbus address 0 Communication configuration is 9600, N, 8,1 (9600bps, no parity bit, 8 data bits, a stop bit) Communication protocol for Modbus-RTU Configuration parameters in EEPROM will not because the module into the "mode" and "will change, when the module is in communication with in the EEPROM configuration parameter is still running mode". |

# 4 Package size, Ordering info, and installation guide

## 4.1 Package size

## 4.2 Selection Order

| Series | Code 1 |  | Measurement parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MS10- | A B |  | Soil volumetric water content (VWC) and temperature Soil volumetric water content (VWC) |  |  |  |  |
|  |  |  | Code 2 |  | The range of volumetric water content of soil |  |  |
|  |  |  | A B C |  | 0-100% 0-50% 0-30% |  |  |
|  |  |  |  |  | Code 3 | Output signal |  |
|  |  |  |  |  | A B C D E F G | Output voltage 0-2V The current output of 4-20mA RS485 interface, Modbus protocol RS485 interface, Modbus protocol and the 0-2V RS485 interface, Modbus protocol and 4-20mA The SDI-12 interface Custom |  |
| Type example： The MS10 series, the soil volumetric water content (VWC) and temperature, soil volumetric water content range of 0-100%, RS485 interface, Modbus protocol. The selection of the code as follows： |  |  |  |  |  |  |  |
| MS10- | A | A |  | C |  |  |  |

## 4.3 Installation Guide

Rapid measurement method: the selected measurement locations, the right to avoid the rocks, to ensure that the needle will not touch the stones like hard object, according to the required depth of cut open the surface soil, maintain the tightness degree of the original soil below the sensor body, clenched vertically inserted into the soil, can not be inserted before and after shaking, ensure the close contact with the soil. A measuring point within a small range test should repeatedly averaging.

Buried in the underground measurement method: vertical drilling diameter greater than 20 cm depth of pit, according to the measurement needs, then the sensor wire inserted into the pit wall in a given level of depth, the pit landfill compaction, ensure the close contact with the soil. Stable after a period of time, can be last for days, months or even longer to measure and record.

If the surface measurement is hard, should first hole (diameter should be less than the diameter of the probe), and then inserted into the soil and the soil compaction and measurement; sensor should prevent violent vibration and impact, but not with a hard object percussion. Because the sensor for black package, in the strong sunlight will make the sensor to make sharp warming (up to over 50 DEG C), in order to prevent the temperature measurement of high temperature impact sensor, please pay attention to sun protection in the field or fields

# 5 The conversion for soil moisture volume percentage，temperature and output

| Model | The parameter range | conversion relation |
| --- | --- | --- |
| Output voltage 0-2V | corresponding temperature of -40-80 ℃ | Temperature =60.0* voltage -40. If the measured voltage is 1.5V, the temperature of =60.0*1.5-40=50.00 ℃. |
|  | volumetric water content corresponding 0-30% | volumetric moisture content=0.15*voltage . If the measured voltage is 1.5V, the volumetric water content was =0.15*1.5= 0.225=22.5%. |
|  | volumetric water content corresponding 0-50% | volumetric moisture content=0.25*voltage . If the measured voltage is 1.5V, the volumetric water content was =0.25*1.5=0.375=37.5%. |
|  | volumetric water content corresponding 0-100% | volumetric moisture content=0.5* voltage . If the measured voltage is 1.5V, the volumetric water content was =0.5*1.5=0.75=75.0%. |
| Current output of 4-20mA | corresponding temperature of -40-80℃ | temperature =7.5 * Current -70 . If the measured current is 10mA, the temperature of =7.5*10-70=5.00 ℃. |
|  | volumetric water content corresponding 0-30% | Volumetric water content = 0.01875 * (current -4). If the measured current is 10mA, the volumetric water content was =0.01875 * (10-4) = 0.1125 =11.25%. |
|  | volumetric water content corresponding 0-50% | Volumetric water content = 0.03125 * (current -4). If the measured current is 10mA, the volumetric water content was =0.03125 * (10-4) = 0.1875 =18.75%. |
|  | volumetric water content corresponding 0-100% | Volumetric water content = 0.0625 * (current -4). If the measured current is 10mA, the volumetric water content was =0.0625 * (10-4) = 0.375=37.5%. |
| RS485 interface, Modbus protocol | corresponding temperature of -40-80℃ | Temperature = temperature register value /100. If the read data is 2013, then the temperature = 2013/100=20.13 C. |
|  | volumetric water content corresponding 0-30% volumetric water content corresponding 0-50% volumetric water content corresponding 0-100% | Volumetric water content = volumetric moisture content register value /10000. If the read data is 1568, then the temperature = 1568/10000=15.68%. |
| Custom |  | The output of models, please contact technical support. |

Note: The unit of voltage formula for V (V), the current unit is ma (mA)

# 6 Modbus Communication Protocol

## 6.1 Serial communication parameters

Modbus is a serial communication protocol, Modicon programmable logic controller (PLC) for the use of published. It has become the industry standard communication protocols, and is now quite common connection between industrial electronic equipment. Modbus has extensive application in the industrial field. Modbus protocol is a master / slave framework agreement. A node is the master node, other nodes using the Modbus protocol in communication from node. Each slave device has a unique address.

Communication parameter default value: baud rate is 9600bps, a start bit, 8 data bits, no parity, one stop bit. Communication protocol for the Modbus RTU protocol. Communication parameters can be set by the program or the Modbus command to change.

6.2 Modbus Register

| parameter name | Register address (16 m) | Parameter type | Modbus Function number | Range of parameters and instructions | default value |
| --- | --- | --- | --- | --- | --- |
|  | Channel 0 |  |  |  |  |
| Temperature | 0x0000 | INT16,read only | 3/4 | -4000-8000 Divided by 100 to obtain the actual temperature value. No temperature measurement model the readout value is always 0. | no |
| Volumetric moisture content | 0x0001 | INT16,read only | 3/4 | 0-10000 Water volume is divided by 10000 to obtain the actual rate. | no |
| Retain | 0x0002 | INT16,read only | 3/4 | Always 0 | no |
| Retain | 0x0003 | INT16,read only | 3/4 | Always 0 | no |
| Volumetric water content was the original AD value | 0x0004 | INT16,read only | 3/4 | Volumetric water content was the original AD value | no |
| Retain | 0x0005 | INT16,read only | 3/4 | Always 0 | no |
| Retain | 0x0006 | INT16,read only | 3/4 | Always 0 | no |
| Retain | 0x0007 | INT16,read only | 3/4 | Always 0 | no |
| Soil Type | 0x0020 | UINT16,read only | 3/6/16 | 0-3 0:Mineral soil (tillage) 1:Sand 2:Clay 3:Organic soil | 0:Mineral soil (tillage) |
| Modbus Address | 0x0200 | UINT16,Read and write | 3/6/16 | 0-255 | 1 |
| baud rate | 0x0201 | UINT16,Read and write | 3/6/16 | 0-7 0:1200bps 1:2400bps 2:4800bps 3:9600bps 4:19200bps 5:38400bps 6:57600bps 7:115200bps | 3:9600bps |
| protocol | 0x0202 | UINT16,Read and write | 3/6/16 | 0~1 0:Modbus RTU 1:Modbus ASCii 2:ADAM Advantech protocol 3:ADAM Advantech protocol (with parity) | 0:Modbus RTU |
| parity | 0x0203 | UINT16,Read and write | 3/6/16 | 0-2 0:No parity 1:Even parity 2:Odd parity | 0:No parity |
| data bits | 0x0204 | UINT16,Read and write | 3/6/16 | 1 1:8 data bits | 1:8 data bits |
| Stop bits | 0x0205 | UINT16,Read and write | 3/6/16 | 0-1 0:1 stop bits 1:2 stop bits | 0:1 stop bits |
| Response delay | 0x0206 | UINT16,Read and write | 3/6/16 | 0-255 The host request command is received after a delay time and response. The delay time for the set value of *10 Ms. When set to 0 without delay. | 0 |
| active output interval | 0x0207 | UINT16,Read and write | 3/6/16 | 0-255 Does not need the host request, to automatically send data fixed time interval. The time interval for the setting value * 1. Set to prohibit active output function 0. | 0 |

UINT16:16 bit unsigned integer register
INT16:16 bit signed integer register
UINT32:32 bit unsigned integer register
UINT32:32 bit signed integer register
BIT: bit register

## 6.3 Detail of Modbus Register

| temperature |  |  |
| --- | --- | --- |
| parameter range | -4000-8000 | Default: none |
| parameter storage | none |  |

Significance: the measured value of the temperature, negative for complement representation.

Example: if the return value is 0702H (16 m, the original code), the first byte byte second byte 07H, low byte is 02H, then the temperature measurement value of (07H*256+02H) /100=17.94 C.

If the return value is FF05H (16 m, complement), the first byte byte second byte FFH, low byte is 05H, then the temperature measurement value ((FFH*256+05H) -FFFFH-1H) /100 = (FF05H-FFFFH-1H) /100=-2.51 C.

| volumetric water content |  |  |
| --- | --- | --- |
| parameter range | 0-5000 | Default: none |
| parameter storage | none |  |

Significance: volumetric water content measurements.

Example: if the return value is 071DH (16 m), the first byte byte second byte 07H, low byte is 1DH, then the volumetric water content was measured (07H*256+1DH) /10000= (7*256+29) /10000=18.21%.

| volumetric water content raw ad |  |  |
| --- | --- | --- |
| parameter range | Not fixed | Default: none |
| parameter storage | none |  |

Significance: volumetric water content was the original AD value.
Example: if the return value is 071DH (16 m), the first byte byte second byte 07H, low byte is 1DH, then the volumetric water content was the original AD value was (07H*256+1DH) = (7*256+29) =1821.

| soil type |  |  |
| --- | --- | --- |
| parameter range | 0-3 | Default: none |
| parameter storage | Immediate memory |  |

Significance: soil type.
0: mineral soil (tillage soil)
1: sand
2: clay
3: organic soil
Example: if the return value is 071DH (16 m), the first byte byte second byte 07H, low byte is 1DH, then the volumetric water content was the original AD value was (07H*256+1DH) = (7*256+29) =1821.

| Modbus Address |  |  |
| --- | --- | --- |
| parameter range | 0-255 | Default:1 |
| parameter storage | Immediate memory |  |

The Modbus address, can be set to 0-255. When the module external address code switch is set to address 0, use this register contents as the slave address. After setting the need to power on or use the RST command to restart module, make this address in force. Use this command to modify the module address does not need to open the cabinet can be set.

| baud rate |  |  |
| --- | --- | --- |
| parameter range | 0-7 0:1200bps 1:2400bps 2:4800bps 3:9600bps 4:19200bps 5:38400bps 6:57600bps 7:115200bps | Default:3 |
| parameter storage | Immediate memory |  |

| protocol |  |  |
| --- | --- | --- |
| parameter range | 0~1 0:Modbus RTU 1:Modbus ASCii | Default:0 |
| parameter storage | Immediate memory |  |

| parity |  |  |
| --- | --- | --- |
| parameter range | 0-2 0:No parity 1:Even parity 2:Odd parity | Default:0 |
| parameter storage | Immediate memory |  |

| data bits |  |  |
| --- | --- | --- |
| parameter range | 1 1:8 data bits | Default: 1, only supports 8 bits of data, the other is invalid |
| parameter storage | Immediate memory |  |

| Stop bits |  |  |
| --- | --- | --- |
| parameter range | 0-1 0:1 stop bits 1:2 stop bits | Default:0 |
| parameter storage | Immediate memory |  |

| response delay |  |  |
| --- | --- | --- |
| parameter range | 0-255 | Default:0 |
| parameter storage | Immediate memory |  |

Serial communication delay in response to a user in the following cases: when the host sends a request command module, delay (RESPONSEDELAY*10) ms, then the response data is returned to the host computer. For example, set the RESPONSEDELAY=5, then the module delay 5*10=50 ms after the response of host requests. When set to 0 for no delay immediate response. This command is mainly applied to the host from the sending State RS485 switch to slow the receiving state occasions.

| active output interval |  |  |
| --- | --- | --- |
| parameter range | 0-255 | Default:0 |
| parameter storage | Immediate memory |  |

Serial communication active output time interval used in the following cases: the host does not need to send a request command, module active output response data, output interval for ACTIVEOUTPUTINTERVAL seconds, such as setting the ACTIVEOUTPUTINTERVAL=5 module, then every 5 seconds according to the communication protocol of the output data set. When set to 0 active output invalid, need a host requests before response. This command is mainly used in GPRS wireless transmission, need to terminal nodes initiatively sends data occasions.

Note: when set to the active output data, RS485 bus can connect a module, in order to avoid data conflict.

## 6.4  CRC16 Validation and Sample Code

For example: 0x0000H-0x0001H read registers, measuring temperature, volumetric water content values
Request: 010300000002 C4 0B (8 bytes)

| Device address | 1 byte | 0x01 |
| --- | --- | --- |
| Function number | 2 bytes | 0x03 |
| starting register address | 2 bytes | 0x0000 |
| number of registers | 2 bytes | 0x0002 |
| Check | 2 bytes | 0xC40B |

Request: 01 03 04 07 02 07 1D E1 7E（9 bytes）

| Device address | 1 byte | 0x01 |
| --- | --- | --- |
| Function number | 1 byte | 0x03 |
| effective number of bytes | 1 byte | 0x04 |
| Data | 4 bytes | 0x07（high temperature byte） |
|  |  | 0x02（low temperature byte） |
|  |  | 0x07（high volume water content byte） |
|  |  | 0x1D（low volume water content byte） |
| check | 2 bytes | 0xE17E |

When the 9 bytes of data receiving equipment return, the following CRC computations, where num (parameter 2) = 9

//-----------------------------------------------------------------------------

//CRC calculation of C51 language function is as follows

/ / input parameters of 1:snd, to check the byte array name

/ / input parameters of 2:num, the total number of bytes to be calibrated. (2 bytes including CRC check)

/ / function return value: return a value other than 0 check failed. Check back 0.

//-----------------------------------------------------------------------------

unsigned int calc_crc16 (unsigned char *snd, unsigned char num)

{

unsigned char i, j;

unsigned int c,crc=0xFFFF;

for(i = 0; i < num; i ++)

{

c = snd[i] & 0x00FF;

crc ^= c;

for(j = 0;j < 8; j ++)

{

if (crc & 0x0001)

{

crc>>=1;

crc^=0xA001;

}

else

{

crc>>=1;

}

}

}

return(crc);

}

To get results back to 0 then the verification is successful, if the validation fails to return to a non-zero value.
After the successful calibration, use the following formula to calculate the temperature and volumetric moisture content (negative to complement representation):
When the temperature = (07H*256+02H) /100=17.94
The volumetric moisture content = (07H*256+1DH) /10000= (7*256+29) /10000=18.21%
If the check fails, the error transfer process, should abandon the collected data, re collection.