- P542.ino  Write program to determine the execution times of various Arduino instructions as shown in the following Table.

| : Arduino UNO R3 : Timer-1 Clock = 16 MHz : |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sn | Instruction | What does the function do? | Execution Time |  |
|  |  |  | Cycles | µs |
| 1 | pinMode(2, INPUT_PULLUP) | Port Line 2 of Port-D is input with internal pull-up | 78 | 4.875 |
| 2 | pinMode(4, OUTPUT); | Port Line 4 of Port-D is output | 70 | 4.375 |
| 3 | DDRD = 0b00001000; |  | 4 | 0.250 |
| 4 | digitalWrite(4, HIGH); | Asserts LH at port line 4 of PORTD | 59 | 3.687 |
| 5 | bitSet(PORTD, 4); | Same as above | 4 | 0.250 |
| 6 | bitWrite(PORTD, 4, HIGH) | Same as above | 4 | 0.250 |
| 7 | digitalWrite(4, LOW); | Asserts LL at port line 4 of PORTD | 61 | 3.812 |
| 8 | bitClear(PORTD, 4); | Same as above | 4 | 0.250 |
| 9 | bitWrite(PORTD, 4, LOW); | Same as above | 4 | 0.250 |
| 10 | bitWrite(PORTD, 4, !bitRead(PORTD, 4)); | Toggles the logic value of port line 4 of PORTD | 8 | 0.500 |
| 11 | bitRead(PORTD, 4); | Reads (from internal latch) the value previously written into port line 4 of PORTD | 3 | 0.187 |
| 12 | digitalRead(2); | Reads logic value from physical pin of port line 2 of PIND. | 36 | 2.250 |
| 13 | bitRead(PIND, 2); | Same as above | 3 | 0.187 |
| 14 | int x = analogRead(A0); | Reads ADC value of Ch-0 and assigns to x. | 3328 | 208.000 |
| 15 | analogReference(DEFAULT); | Sets the Reference value (5V) for ADC | 4 | 0.250 |
| 16 | analogWrite(3, 0x23); | Direct value for pulse width of PWM | 94 | 5.875 |
|  | analogWrite(3, OCR2B); | Same as above, but value comes from a register | 153 | 9.562 |
| 17 | analogWrite(3, analogRead(A0)); | Same as above, but value comes from ADC | 3477 | 217.312 |

Methodology used: The procedure involves the following tricks: start T1 at 16 MHz clock before the execution of the ‘target’ instruction; execute the target instruction; stop the T1 after the execution of the target instruction; show the clock accumulated by TCN1 on LCD.