Water tank /                               water motor                             water flow sensor

Water sump                                   pump                                            yf-s201

Relay                                                                                            water

Arduino Uno                                         outlet

RFID Reader/writer                                                                                                                                                                                                                                      mfrc522                                             16*2 Lcd display

CIRCUIT INTERFACE

RFID READER/WRITTER:                                   ARDUINO UNO:

3.3V                                                                      3.3V

GND                                                                      GND

RST                                                                       DIGITAL-9

SDA                                                                       DIGITAL-10

SCK                                                                        DIGITAL-13

MOSI                                                                     DIGITAL-11

MISO                                                                      DIGITAL-12

LCD DISPLAY(16*2):                                            ARDUINO UNO:

VSS                                                                         GND

VCC                                                                        5V

VO                                                                          GND

RS                                                                           DIGITAL-7

RW                                                                         GND

E                                                                             DIGITAL-6

A                                                                             5V WITH 10K RESISTOR

K                                                                             GND

D4                                                                          DIGITAL-5

D5                                                                          DIGITAL-4

D6                                                                          DIGITAL-3

D7                                                                          DIGITAL-2

WATER FLOW SENSOR(YF-S201):                    ARDUINO UNO:

RED                                                                       5V

BLACK                                                                   GND

YELLOW                                                               DIGITAL-8

RELAY:                                                                 ARDUINO UNO:

5V                                                                        5V

GND                                                                    GND

IN1                                                                      A0

REQUIREMENT:

- Initially a welcome text is to be displayed on the lcd display and delay 1sec there after a text: please place your card is to be displayed and should remain until the card is place on the RFID reader/writer.

- The RFID reader/writer should verify the card placed and if it is valid the lcd should display the text as “processing” or else it should display “invalid card” and the RFID reader and writer should send the respective signal to the Arduino.

- Arduino after receiving signal from RFID reader/writer should send signal to relay and the relay turns on the water motor pump.

- The water flow sensor connected to water motor pump outlet measures the quantity of water and after the flow of respective quantity of water the Arduino should again send signal to relay to off the water motor pump.

- At the end the lcd should display completed and delay 1sec and should display thank you .

- The system should get reset.

- Each RFID card should be usable only twice a day.

- RFID card will get restored for every 24hrs.